REDSTONE Machinery
charlene@redstonemachinery.net
 (647) 705-3487 cell

(905) 886-3487 office

[image: image1.jpg]



#152CN – Ligmatech Through Feed Case Clamp and Fox Ring Wrapper (available without ring wrapper):
Feed through Ligmatech case clamp line with Fox ring wrapper and corresponding conveyors. Ligmatech clamp, model MPP100/10, through feed, pneumatic, separately adjustable vertical and horizontal pressure, working direction left to right, workpiece parameters: carcass dimensions: length min. 300mm, max 2,500mm, height min 300mm, max 1,200mm, depth min 300mm, max 1,000mm, working height 500mm, conveying speed min. 8m/min, max 17 m/min, capacity approx. 2 cycles/min, forming pressure: vertically max 17kN, horizontally max 16 kN, weight 4 t, required room height 4,000mm, PLC system with alphanumeric input for the simple and convenient operation and programming, connected load approx. 3kW, switch cabinet attached, control panel on swivelling arm, assembly table infeed belt 2,700mm long.  FOX R360 automatic ring wrapper, to accommodate product max width 48", max height 32", max length 36" and up, controls: main panel NEMA 12 (36"X36" cabinet on each unit), Mitsubishi PLC, 575 volt, main fuses 20A. Ring: 60” ID Slewing Ring Gear, belt driven, high speed, ring will support up to 100,000 lbs., machined urethane ring support wheel with two sealed bearings (qty of 10), 12” dual urethane drive wheels with 1-15/16” axles, #60 chain and sprocket drive, ring speed from 0 to 60rpm, pass through height of product 43.5”, ring support frame 3/8” steel plate reinforce with 3”x3”x3/16” HSS tube. Mechanics: pneumatic film cut with hot wire impulse cutter, heavy duty 3”x3”x3/16” HSS tubing frame construction, fully guarded for operator protection, standard name brand parts which can be obtained from local supplier. InFeed and Exit Roller: 10’ long belt conveyor at infeed and 10’ long belt conveyor at exit, 65” wide rough top belt, 1HP 90V DC motor, speed 0 – 15 fpm, soft start/stop control, auto indexing of loads by photo eyes.  Film Carriage: 10” FOX “LINEAR” pre-stretch film carriage, supports film rolls 10” long by 10” diameter, easy fill tensioning adjustment on carriage control, 1/2HP DC 90 V motor film delivery drive, power provided to film carriage by slip rings, film prestretch set at 200% by factory. ******Available without the ring wrapper for cheaper price. 
Selling Price: $85,000.

USED MACHINERY IS SOLD ON AN “AS IS” BASES AND IS NOT COVERED UNDER WARRANTY.
